ALGERIAN DEMOCRATIC AND POPULAR REPUBLIC
MINISTRY OF HIGHER EDUCATION AND SCIENTIFIC RESEARCH
ABBAS LAGHROUR UNIVERSITY OF KHENCHELA
Faculty of Science And Technology
Department of Informatics
Master 2 Ai

Subject:

Smart Grid Energy Management System - Technical Report

Presented By:

AOUAIDJIA Mouna

CHAHAT Rima

Executive Summary

This project implements an IoT-based Smart Grid Energy Management System with real-time monitoring, cloud storage, and AI-powered energy consumption prediction. The system integrates hardware sensors, MQTT communication protocol, MongoDB cloud database, Django web framework, and Machine Learning algorithms to create a comprehensive solution for modern energy grid management.

Table of Contents

- System Architecture Overview

  - Hardware Layer - IoT Sensors

  - Communication Layer - MQTT Protocol

  - Gateway Layer - Regional Data Aggregation

  - Cloud Storage - MongoDB Atlas

  - Backend - Django Web Framework

  - Frontend - Interactive Dashboard

  - AI/ML Module - Energy Prediction

- System Operation & Execution

1-1-System Architecture Overview

Layer 5: User Interface (Web Dashboard)

Layer 4: Application Logic (Django)

Layer 3: Cloud Storage (MongoDB Atlas)

Layer 2: Gateway & Communication (MQTT)

Layer 1: IoT Sensors (Physical Devices)

The system is built to monitor five distinct urban regions: Commercial, Downtown, Hybrid, Port, and Residential. Each region operates independently at the hardware level but is unified through centralized cloud infrastructure, allowing for both localized monitoring and city-wide analytics.

1-2. Hardware Layer - IoT Sensors

The hardware layer forms the foundation of our monitoring system, consisting of three primary sensor categories deployed across the five regions:

Smart Meters (Energy Monitoring)

Each region is equipped with 100 smart meters (500 total across all regions), strategically distributed to monitor residential and commercial buildings. These devices continuously measure:

- Voltage (V) - Real-time electrical potential monitoring

- Current (A) - Load current measurement

- Power (kW) - Instantaneous power consumption

- Energy Consumed (kWh) - Cumulative energy usage

- Power Factor - Efficiency metric for electrical systems

The smart meters operate on a 5-second sampling interval, ensuring near real-time data capture. This high-frequency sampling is critical for detecting anomalies such as power surges, voltage drops, or unusual consumption patterns that might indicate equipment malfunction or energy theft.

Feeder Monitoring Units

Each region has a dedicated feeder monitoring system that tracks the main power distribution lines. These units measure:

- Load Current (A) - Total current flowing through the feeder

- Load Voltage (V) - Supply voltage stability

- Frequency (Hz) - Grid frequency for power quality assessment

- Power Factor - Overall efficiency of the distribution network

Feeder monitoring is essential because it provides a macro-level view of grid health. Unlike individual smart meters that track consumption at the building level, feeders give us insight into the distribution infrastructure itself, helping identify bottlenecks or overloaded circuits before they cause outages.

Environmental Sensors

To correlate energy consumption with environmental factors, each region is equipped with three types of environmental sensors:

- Temperature Sensors - Measuring ambient temperature in °C

- Humidity Sensors - Tracking relative humidity (%)

- Wind Speed Sensors - Monitoring wind conditions (m/s)

These environmental metrics are crucial for understanding external factors affecting energy demand. For example, high temperatures typically correlate with increased air conditioning usage, while humidity levels impact heating efficiency. This data enables predictive modeling and helps utility companies anticipate demand spikes.

1-3. Communication Layer - MQTT Protocol

The communication layer is built entirely on the MQTT (Message Queuing Telemetry Transport) protocol, chosen specifically for its lightweight nature and suitability for IoT applications. MQTT operates on a publish-subscribe model, which offers several critical advantages over traditional request-response protocols:

Why MQTT?

- Bandwidth Efficiency: MQTT has a minimal overhead (as small as 2 bytes per message), making it ideal for devices with limited connectivity.

- Quality of Service (QoS) Levels: Supports three QoS levels (0, 1, 2), allowing us to balance between delivery guarantees and network efficiency.

- Bi-directional Communication: Enables both data collection from sensors and remote control commands to devices.

- Session Persistence: Maintains connections even during brief network disruptions, ensuring data integrity.

Topic Structure

The MQTT (mosquito) topic hierarchy is designed for clarity and scalability:

city/{region}/{device_type}/{device_id}

Examples:

- city/commercial/smartmeter/1 - Smart meter data from house 1 in the commercial region

- city/downtown/feeder/feeder_downtown - Feeder data from downtown region

- city/hybrid/env/temperature - Temperature sensor data from hybrid region

This hierarchical structure allows gateways to subscribe to specific regions (e.g., city/commercial/#) while the cloud system can subscribe to all data (city/#). The wildcard subscriptions (#) enable flexible data routing without hardcoding device addresses.

Message Format

All sensor data is transmitted in JSON format for human readability and easy parsing:

{

"device_type": "smart_meter",

"house_id": "commercial_house_1",

"data": {

"voltage": 220.5,

"current": 5.2,

"power": 1.15,

"energy_consumed": 25.3,

"power_factor": 0.92

},

"timestamp": "2025-01-15T10:30:00Z"

}

1-4. Gateway Layer - Regional Data Aggregation

The gateway layer serves as an intelligent intermediary between the hardware layer and the cloud infrastructure. Each of the five regions has a dedicated gateway that performs local data aggregation, validation, and preprocessing before forwarding data to the cloud.

Gateway Responsibilities

Data Collection and Validation Each gateway subscribes to its region-specific MQTT topics and performs real-time data validation:

- Type Checking: Ensures voltage readings are within expected ranges (190-250V)

- Timestamp Verification: Validates that sensor timestamps are synchronized

- Duplicate Detection: Filters out redundant messages from sensors

Data Enrichment Gateways automatically append metadata to incoming sensor data:

- Region Name: Tags each message with its origin region (commercial, downtown, etc.)

- Received Timestamp: Records when the gateway processed the message

- Topic Information: Preserves the MQTT topic for debugging and routing

Feeder Data Aggregation One unique challenge in the system is that feeders send their measurements in separate messages for different parameters (current, voltage, frequency, power factor). The gateway intelligently aggregates these separate messages into a single comprehensive document.

1-5. Cloud Storage - MongoDB Atlas

The cloud storage layer uses MongoDB Atlas, a fully managed cloud database service, hosted on AWS infrastructure. The choice of MongoDB over traditional relational databases (SQL) was driven by several factors:

Why MongoDB?

Schema Flexibility: IoT sensors often evolve, adding new metrics or changing data formats. MongoDB's document-based model allows us to store varying sensor structures without schema migrations.

Horizontal Scalability: MongoDB's sharding capabilities enable us to distribute data across multiple servers as the system grows from 500 to potentially 50,000 sensors.

Time-Series Optimization: While we're not using MongoDB's native time-series collections (due to aggregation requirements), the document model efficiently handles timestamped sensor readings.

Aggregation Framework: MongoDB's powerful aggregation pipeline enables complex analytics directly in the database, such as calculating regional averages or identifying consumption trends.

Database Structure

The database contains three primary collections, each serving a specific purpose:

smartmeters Collection Stores individual house-level energy consumption data:

{

"_id": ObjectId("..."),

"house_id": "commercial_house_1",

"device_type": "smart_meter",

"region_name": "commercial",

"data": {

"voltage": 220.5,

"current": 5.2,

"power": 1.15,

"energy_consumed": 25.3,

"power_factor": 0.92

}

"timestamp": "2025-01-15T10:30:00Z",

}

feeders Collection Aggregates feeder-level distribution metrics:

{

"_id": ObjectId("..."),

"feeder_id": "feeder_commercial",

"device_type": "feeder",

"region_name": "commercial",

"data": {

"load_current": 125.5,

"load_voltage": 225.0,

"frequency": 50.1,

"power_factor": 0.95

},

"timestamp": "2025-01-15T10:30:00Z"

}

env_sensors Collection Stores environmental monitoring data:

{

"_id": ObjectId("..."),

"device_type": "env_sensor",

"sensor_type": "temperature",

"region_name": "commercial",

"data": 24.5,

"unit": "celsius",

"timestamp": "2025-01-15T10:30:00Z"

}

1-6. Backend - Django Web Framework

The backend is built using Django, a high-level Python web framework known for its "batteries-included" philosophy. Django was chosen for several compelling reasons:

Why Django?

- Rapid Development: Django's ORM, admin interface, and built-in authentication accelerate development.

- Security: Built-in protection against SQL injection, CSRF, XSS, and clickjacking.

- Scalability: Powers high-traffic sites like Instagram and Pinterest.

- Pymongo Integration: Seamless integration with MongoDB through the pymongo library.

1-7. Frontend - Interactive Dashboard

The frontend is a single-page application (SPA) built with vanilla HTML5, and CSS3, without relying on heavy frameworks like React or Angular. This decision was deliberate:

Design Philosophy

- Performance: No framework overhead means faster initial page loads

- Simplicity: Easier to debug and maintain for IoT-specific use cases

Dashboard Components

Main Dashboard (home.html) The landing page provides a bird's-eye view of the entire grid:

- Smart Meters Card: Displays latest 10 house readings with region badges

- Feeders Card: Shows distribution network status

- Environmental Sensors Card: Current temperature, humidity, wind speed

- Regions Card: Summary of all five regions

Each card auto-refreshes independently, and the user sees a visual indicator showing the last update time.

Houses Details Page This page offers deep insights into individual house consumption:

Four Interactive Charts:

- Power Distribution (Doughnut Chart): Categorizes houses into Low/Medium/High/Alert consumption tiers

- Top 10 Houses (Bar Chart): Identifies highest consumers for targeted efficiency programs

- Energy vs Power Scatter Plot: Reveals correlation between instantaneous power and cumulative energy

- Voltage Distribution (Line Chart): Monitors grid voltage stability across houses

1-8-the Machine Learning Part (Random Forest Model)

In this project, I used Random Forest, a supervised machine learning algorithm, to build a predictive model capable of learning patterns from the dataset and generating accurate predictions.

Why Random Forest?

Random Forest is a robust ensemble method that:

- Combines multiple decision trees to reduce overfitting.

- Improves prediction stability by averaging the outputs of many trees.

- Handles noisy data and non-linear relationships effectively.

- Works well even when features vary in scale or distribution.

How It Works

- The algorithm creates many random decision trees.

- Each tree is trained on:

  - A random sample of the data (bootstrapping).

  - A random subset of features at each split.

- The model then:

  - Aggregates the predictions of all trees (majority vote for classification, average for regression).

- This ensemble architecture makes the model:

  - More generalizable

  - Less sensitive to outliers

  - More accurate than a single tree

2-System Operation & Execution

Prerequisites

- Make sure all required Python packages are installed:

pip install -r requirements.txt

1. Connect to the Database

To connect the system to the MongoDB Atlas cloud database, use one of the following connection strings:

"mongodb+srv://AdaLovelace:AdaLovelace1817@cluster0.jfdolkd.mongodb.net/SmartGrid?retryWrites=true&w=majority"

Or

"mongodb+srv://AdaLovelace:AdaLovelace1817@cluster0.jfdolkd.mongodb.net/SmartGrid"

If you encounter issues connecting to the cloud, you can create a new MongoDB Atlas account and use a personal connection string, for example:

mongodb+srv://Dr_messaoudi:123test@cluster0.jfdolkd.mongodb.net/SmartGrid?retryWrites=true&w=majority

2-run the dashboard

Navigate to the Django frontend folder and start the server:

cd frontend

cd SmartGridWeb

python manage.py runserver

After running the command, open your browser and visit the local URL (typically http://127.0.0.1:8000/). You will see the dashboard interface where you can interact with live data and AI predictions.

3. Optional: Update Data Automatically

To have live updates on the dashboard every 5 minutes, you can run the system in the following order:

- Gateways – Start the regional gateway scripts to aggregate and normalize data from sensors.

- Cloud Receiver – Run the cloud receiver script to forward data to MongoDB Atlas.

- Sensors – Run the IoT sensor scripts to continuously publish data.

Once all components are running, the dashboard will automatically refresh with updated metrics and predictions every 5 minutes.

In these pages you can:

- View live energy data from all sensors.

- Inspect region-based summaries (residential, commercial, downtown, hybrid, port).

- Access house-level consumption statistics.

- Check AI-powered energy predictions for upcoming hours.

- Explore interactive charts and tables for detailed insights.

- Send reports via email directly from the dashboard.

Additional pages provide statistical visualizations for sensors, regions, and individual houses. These pages are accessible once you enter the dashboard, allowing you to analyze the collected data and AI predictions in depth.

The prediction page:

You can even send the results via an email and get the following: